EXP. NO: 9

PERFORMANCE EVALUATION OF AM GENERATION AND

DETECTION UNDER NOISY CHANNEL CONDITIONS USING

MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal, introduce Additive White Gaussian Noise

(AWGN), and analyze the effect of noise on the transmitted signal using MATLAB.

ii) To recover the original message signal from the noisy AM signal using envelope detection

and compare the transmitted and recovered waveforms to evaluate the performance of the AM

communication system.

PROGRAM (MATLAB CODE):

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500;

fc = 5000;

% Message Signal

m = cos(2*pi*fm*t);

c = cos(2*pi*fc*t);

% AM Modulation

am = (1 + m).*cos(2*pi*fc*t);

% Add AWGN Noise

noisy = awgn(am,10,'measured'); % 10 dB SNR

% Envelope Detection

env = abs(hilbert(noisy));

% Display Results

figure;

subplot(5,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,am)

title('AM Signal')

grid on

subplot(5,1,4)

plot(t,noisy)

title('AM Signal with AWGN Noise')

grid on

subplot(5,1,5)

plot(t,env)

title('Recovered Signal')

grid on

OUTPUT(GRAPH):