EXP. NO: 10

PERFORMANCE ANALYSIS OF FM SYSTEM UNDER GAUSSIAN

WHITE NOISE USING MATLAB

1. To generate a Frequency Modulated (FM) signal in MATLAB using the specified carrier and

message signal parameters.

2. To introduce Additive White Gaussian Noise (AWGN) into the generated FM signal and simulate

the effect of a noisy communication channel using MATLAB.

3. To analyze the effect of AWGN on the transmitted FM signal by comparing the original and noisy

FM signals using time-domain and frequency-domain analysis.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500; % Message frequency

fc = 5000; % Carrier frequency

kf = 100; % Frequency sensitivity

% Message Signal

m = cos(2*pi*fm*t);

c = cos(2*pi*fc*t);

% FM Modulation

fm_sig = cos(2*pi*fc*t + 2*pi*kf*cumsum(m)/fs);

% Add Gaussian White Noise

noisy = awgn(fm_sig,10,'measured'); % 10 dB SNR

% Display

figure;

subplot(4,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(4,1,2)

plot(t,m)

title('Carrier Signal')

grid on

subplot(4,1,3)

plot(t,fm_sig)

title('FM Signal')

grid on

subplot(4,1,4)

plot(t,noisy)

title('FM Signal with Gaussian White Noise')

grid on

OUTPUT(GRAPH):