EXP. NO: 17

PERFORMANCE EVALUATION OF DIGITAL COMMUNICATION

SYSTEMS USING EYE DIAGRAM ANALYSIS UNDER ADDITIVE

WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

Objectives

1. To generate a digital communication signal in MATLAB and transmit it through an

Additive White Gaussian Noise (AWGN) channel.

2. To obtain and analyze the eye diagram of the received digital signal for different noise

conditions.

3. To measure eye-opening parameters such as eye height, eye width, crossing level and

timing variation.

4. To study the effect of decreasing SNR / Eb/N0 on inter-symbol interference, noise margin

and timing performance.

5. To compare the transmitted and received signal characteristics and evaluate

communication-system performance.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

%% Parameters

N = 100; % Number of bits

sps = 20; % Samples per symbol

SNR = 10; % Signal-to-Noise Ratio (dB)

%% Generate Random Binary Data

data = randi([0 1],1,N);

%% NRZ Encoding (0 -> 0, 1 -> 1)

tx_signal = repelem(data,sps);

%% Pass Signal through AWGN Channel

rx_signal = awgn(tx_signal,SNR,'measured');

%% Time Axis

t = (0:length(tx_signal)-1)/sps;

%% Plot Results

figure;

% Original Binary Data

subplot(4,1,1)

stairs(data,'LineWidth',1.5)

grid on

title('Original Binary Data')

xlabel('Bit Number')

ylabel('Bit')

ylim([-0.2 1.2])

% Transmitted NRZ Signal

subplot(4,1,2)

plot(t,tx_signal,'LineWidth',1.5)

grid on

title('Transmitted NRZ Signal')

xlabel('Time')

ylabel('Amplitude')

ylim([-0.5 1.5])

% Received Signal after AWGN

subplot(4,1,3)

plot(t,rx_signal,'r')

grid on

title(['Received Signal with AWGN (SNR = ',num2str(SNR),' dB)'])

xlabel('Time')

ylabel('Amplitude')

% Eye Diagram

subplot(4,1,4)

eyediagram(rx_signal,2*sps)

title('Eye Diagram of Received Signal')

OUTPUT(GRAPH):